# DVM Discovery

# Hands-On & Flexible Learning of 
Digital Voltmeter Circuits

Contents

1	Intro	2

2	Overview	2

3	Applications	3

4	Block Diagram	3

5	Simplified Signal Flow	3

6	Gain Stages vs Ranges	4

7	Schematic	5

8	Voltage Range Setup	11

9	DVM Basic - 2 Ranges, Unipolar Input	12

10	DVM Advanced - 3 Ranges, Bipolar Input	15

11	Make it!	18

12	Bonus Sections	22

# Intro

The DVM Discovery Board allows students, engineers and DIYers to quickly build and experiment with a variety of Digital Voltmeter Circuits. Hands-on learning of real-world circuits boosts understanding and intuition!

- Easy-to-use experimenter board

- Solderless headers allow quick component changes

- Multiple circuit design options

  - 1 to 3 Customizable Voltage Ranges

  - Unipolar (+ input only) or Bipolar (+/- input)

  - Range Select: Manual or Auto

  - Precision Mode (voltage reference option)

- Simple Interface to uController Boards (ex: Arduino UNO)

- Excel Toolkit calculates component values, ranges and resolution.

# Overview

The DVM Discovery provides flexible circuitry for a multi-range voltmeter and easy uController interface. You can quickly build and explore many measurement options.

YELLOW boxes show solderless headers for easy component changes!

- Input Divider:  Standard & High-Voltage Range option

- Amplifier:  Low-Voltage Range option

- Negative Rail:  Bipolar Input option

- Voltage Reference:  Precision Mode option

- Push-Button Range Select

- LED Indicators - Digital Output Level

# Applications

Here’s just a few of the real-world measurements needed across the ranges,

- 1V Range - diodes, pressure / temperature sensors, current sense voltages.

- 5V Range - 1.5V household batteries, 3.3V / 5V supplies, 4V power-tool batteries.

- 25V Range - 9V household / 12V auto batteries, 15V supplies, 18V solar cell

NOTE: You can customize the 1V / 25V ranges for your application (48V eBike batteries for example). See Make It! Section.

# Block Diagram

The basic blocks allow for flexible build / design options. Customize for functionality as well as your learning goals.

# Simplified Signal Flow

The simplified diagram below shows the main path of the measured signal.

# Gain Stages vs Ranges

How can you achieve three voltage ranges? The RED shading shows CLOSED switches.

# Schematic

The DVM Discovery is created using straightforward circuits and standard parts. They provide practical real-world applications of classic circuits. You can also deep dive into the theory of these foundational circuits as well.

NOTE: For full schematic, see Bonus Sections.

## Input Divider, Analog Mux, Low-Pass Filter

Input Voltage Divider

- The R1, R2 Divider scales down high voltages to the +5V max of the ADC input.
   Kdiv = V1 / Vin = R2 / (R1+R2)

Analog Mux

- Solid-State Relays U4 and U5 create an Analog Mux

  - Normal Voltage Range: U4 connects Vin directly to V1.

  - High Voltage Range: U5 connects the Divider tap to V1.

Low-Pass Filter

- R3, C1 create a low-pass filter: fc = 1 / (2 * pi* R3 * C1)

- Reduces unwanted 60Hz, radio interference and protects against input spikes.

## Amplifier with Selectable Gain

You can configure U1 as a selectable gain amplifier or a unity-gain buffer.

Selectable Gain Amplifier

- Unity Gain Amplifier - U6 Switch ON

  - U6 shorts U1 output to negative input.

- Gain Amplifier - U6 Switch OFF

  - Ka = V3 / V2 = R6 / R5 + 1

Unity-Gain Amplifier

- U6 not used.

- R5 = short (or < 100 ohms)

- R6 = open

Input Protection

- Schottky Diodes D4, D5 limit V2 to within VCC and VEE keeping U1’s pins under their MAX voltage input rating.

## Scale & Shift, Buffer

You can scale and shift a bipolar input (-5V to 5V) to a unipolar one (0V to 5V) using a simple two resistor network R7, R8!

Scale and Shift

Kscale=V4V3=R8R7+R8

Voffset=VREF0∙R7R7+R8

Unipolar Input Mode – No Scale or Shift

- Input:    V3 = 0 to 5V Range

- Output: V4 = 0 to 5V Range

  - R7 = 1k, R8 = open

  - Scale:  Kscale = 1 (Unity Gain)

  - Shift:  Voffset = 0V

Bipolar Input Mode - Scale & Shift

- Input:    V3 = -5V to 5V Range

- Output: V4 = 0 to +5V Range

  - R7 = 10k, R8 = 10k, VREF0 = +5.0V

  - Scale:  Kscale = R8 / (R7+R8) = 0.5

  - Shift:  Voffset = Vref0 * R7 / (R7+R8) = +2.5V

  - V4 = V3*0.5 + 2.5V

Optional Low-Pass Filters

- R7||R8 and C2:  fc = 1 / (2 * π * R7||R8 * C2)

- R9 and C3:  fc = 1 / (2 * π * R10 * C3)

## Buffer and External Interface

Output Buffer

- U1B buffers the medium impedance at node V4 and drives the Microcontroller’s ADC input.

- R9 can provide isolation from capacitive load if driving long cables / wires to ADC input.

- C3 (and R9) can provide addition low-pass filtering.

Microcontroller Interface

- Connector J2 allows easy access to external controller or computer.

| Signals | Controller Signal | Description |
| --- | --- | --- |
| VCC, GND | Power | Power for DVM PCB |
| D1, D2, D3 | Digital Outputs | Solid-State Switch Control Low State – Turns ON U4, U5, U6 |
| D4 | Digital Input | Push-Button State for Range Select. |
| ADC | Analog Input A0 | Voltmeter Reading |
| VREF | Analog Input (A1) | Reference Voltage for Precision Mode |

Pushbutton

- SW1 for external input to manually select Vrange.

LED Indicators - Digital LO & Switch ON

- LEDs (D1,D2,D3) light up when digital outputs (D1,D2,D3) are driven LO turning Opto-Switches (U4,U5,U6) ON.

## Negative Rail

You’ll need a negative supply to power U1 for bipolar inputs.

- U2 is a Switched-Capacitor Converter that creates a negative voltage (VEE) from a positive input voltage (VCC).

- U2’s internal switches alternatively charge C15 to VCC then deliver the charge to C16.

## Voltage Reference

A precise voltage reference enables you to measure in Precision Mode.

- U7 is an IC that behaves like a precision 2-terminal Zener Diode.

- VREF0 drives the Scale and Shift Network

  - VREF0 = VCC = +5V (JP1: 1-2)

  - VREF0 = VREF = +4.096V (JP1: 2-3)

Precision Mode Options

- Connect VREF to an ADC input.
You can measure this precision voltage and apply a Gain correction to all VIN readings.

- Connect VREF directly to the Vref pin of the controller’s ADC.
The ADC automatically scales the measurements to this reference.

## Input Test Levels

A simple resistor string provides 3 handy on-board voltages for testing multiple ranges. For RT1 = RT2 = 10k and RT3 = 1k you get the following levels.

- VCC = +5V Supply.

- VT1 = VCC * (RT2+RT3) / (RT1+RT2+RT3)
        = 2.619V

- VT2 = VCC * (RT3) / (RT1+RT2+RT3)
        = 0.238V

Create any two test voltages that you might need!

# Voltage Range Setup

## Ranges (Default)

| Range | Vrange | Vrange Max | Overrange (%) |
| --- | --- | --- | --- |
| Standard | 4V | 5V | 25% |
| High | 20V | 25V | 25% |
| Low | 1V | 1.25V | 25% |

## Voltage Range, Gains and Digital Outputs

| Voltage Range | Divider Gain | Amplifier Gain | Analog Switches | Digital Outputs |
| --- | --- | --- | --- | --- |
| Standard | 1.0 | 1.0 | U4 = ON U6 = ON | D1 = LO D3 = LO |
| High | R2 / (R1+R2) | 1.0 | U5 = ON U6 = ON | D2 = LO D3 = LO |
| Low | 1.0 | R6 / R5 + 1 | U4 = ON | D1 = LO |

NOTE: LED’s D1, D2, D3 turn ON when digital outputs D1, D2, D3 go LO!

## Signal Levels

| Polarity | VIN | V4 | VADC |
| --- | --- | --- | --- |
| Unipolar | 0V to +Vrange_max | 0V to +5V | 0V to +5V 0V at VIN = 0 |
| Bipolar | -Vrange_max to +Vrange_max | -5V to +5V | 0V to +5V 2.5V at VIN = 0 |

## Software Calculation of VIN

| Polarity | VIN | Ktot | Calc VIN from ADCword |
| --- | --- | --- | --- |
| Unipolar | 0 to +Vrange_max | Kdiv*Kamp | VIN = (ADCword / 1024 * VREF) / Ktot |
| Bipolar | -Vrange_min to +Vrange_max | Kdiv*Kamp*Kscale | VIN = (ADCword / 1024 * VREF – VREF / 2) / Ktot |

# DVM Basic - 2 Ranges, Unipolar Input

Let’s take our DVM for a test drive! Build this voltmeter to get some hands-on experience with multi-ranges. You’ll get to apply some real-world testing of your house-hold batteries.

Description

- Ranges: +4V, +20V

- Input:  Unipolar (Pos input only)

- Range Select:  Manual Push-Button

- Low-Pass Filter:  8 Hz

- Overrange:  +5V, +25V (25%)

Simplified Schematic

Install Components on PCB (See DVM Basic in Make it! Section)

Install Components on 4-Pin Solderless Headers

- R1 = 825k, R2 = 200k  (Input Divider)

- R3 = 200k, C1 = 0.1uF  (Low-Pass Filter)

- R4 = 1k  (connect to VADC)

Option: Solder components above directly on PCB without headers.

Connect Arduino UNO to DVM Discovery

| Arduino | Description | Discovery | Description |
| --- | --- | --- | --- |
| A0 | Analog ADC Input | VADC | Measured Voltage Signal |
| D11, D12 | Digital Outputs | D1, D2 | Opto-Switch Control and LEDs |
| D4 | Digital Input | D4 | Push-Button Logic State |
| +5V, GND | Power | VCC, GND | Power for Switches, Amp, etc. |

Check Out

- Connect Arduino USB to PC or Laptop

- Use a DVM to verify +5.0V between Testpoints VCC and GND.

Run Code

- Download the file:  DVM_discovery_basic_1.ino

- Start the Arduino IDE and Open the file listed above.

- Hit the Upload and Run buttons.

- Press pushbutton repeatedly and verify that LEDs D1 and D2 alternately turn ON and OFF.

Test Voltage

- Press the push-button to select the 4V Range.
  LEDs:  D1 = ON, D2 = OFF

- Connect the VIN Test Clip (RED) to VT1 (2.69V).

- Read voltage at Arduino Serial Monitor on the PC screen.

- Very the reading is within +/-0.05V

- Press the push button to select the 20V Range.
  LEDs:  D1 = OFF, D2 = ON

- Read voltage.

- Very the reading is within +/-0.1V

Test 1.5V and 9V Batteries

- Connect a 1.5V or 9V battery’s + and -leads to VIN and GND.

- Select the 4V or 20V Range.

- Check 1.5V health - between 1.1V and 1.5V

- Check 9V health - between 7.5V and 9V.

# DVM Advanced - 3 Ranges, Bipolar Input

Let’s extend the capabilities of our DVM to 3 Voltage Ranges and measure both 
+/- input polarities!

Description

- Ranges: ±1V, ±4V, ±20V

- Input:  Bipolar (Pos & Neg Voltages)

- Range Select:  Manual Push-Button

- Low-Pass Filter:  8 Hz

- Overrange:  ±1.25V, ±5V, ±25V (25%)

Simplified Schematic

Install Components on PCB (See DVM Advanced in Make it! Section)

Install Components on 4-Pin Solderless Headers

- R1 = 825k, R2 = 200k  (Input Divider)

- R3 = 200k, C1 = 0.1uF  (Low-Pass Filter)

- R5 = 10k, R6 = 30.1k  (Amplifier)

- R7 = 10k, R8 = 10k  (Scale and Shift)

- R9 = 1k (connect to VADC)

Option: Solder components above directly on PCB without headers!

Jumpers

- JP1:1-2  (VREF0 = +5V)

Connect Arduino UNO to DVM Discovery

| Arduino | Description | Discovery | Description |
| --- | --- | --- | --- |
| A0 | Analog ADC Input | VADC | Measured Voltage Signal |
| D11, D12, D13 | Digital Outputs | D1, D2, D3 | Opto-Switch Control and LEDs |
| D4 | Digital Input | D4 | Push-Button Logic State |
| +5V, GND | Power | VCC, GND | Power for Switches, Amp, etc. |

Check Out

- Connect Arduino USB to PC or Laptop

- Use a DVM to verify +5.0V between Testpoints VCC and GND.

- Verify -5.0V between VEE and GND.

- Verify +4.906V between VREF and GND.

Run Code

- Download the file:  DVM_discovery_3range_bipolar_1.ino

- Start the Arduino IDE and Open the file listed above.

- Hit the Upload and Run button.

- Press pushbutton repeatedly and verify that LEDs D1, D2, D2 turn ON and OFF.

4V Range

- Press the push-button to select the 4V Range.
  LEDs:  D1 = ON, D2 = OFF, D3 = ON

- Connect the VIN Test Clip (RED) to VT1 (2.619V).

- Read voltage at Arduino Serial Monitor on the PC screen.

- Verify the reading is within +/-0.05V

20V Range

- Press the push button to select the 20V Range.
  LEDs:  D1 = OFF, D2 = ON, D3 = ON

- Connect the VIN Test Clip (RED) to VT1 (2.619V).

- Verify the reading is within +/-0.1V

1V Range

- Press the push button to select the 1V Range.
  LEDs:  D1 = ON, D2 = OFF, D3 = OFF

- Connect the VIN Test Clip (RED) to VT2 (0.238V).

- Verify the reading is within +/-0.05V

# Make it!

Here’s the parts list for the Basic and Advanced DVMs.

- Modify and build for your own learning goals.

- Install only parts needed for your design.

- All Resistors: 1/4W, 5% unless noted.

- All Capacitors: Ceramic, >25V

You can customize the high and low ranges for your application.

- High Range (25V default) – adjust R1, R2

- Low Range (1V default) – adjust R6, R5

## DVM Basic - 2 Ranges, Unipolar Input

PASSIVES

| R1 | 1 | 825k | 1% for precision | Install group on 4-pin headers |
| --- | --- | --- | --- | --- |
| R2 | 1 | 200k | 1% for precision |  |
| R3 | 1 | 200k |  |  |
| C1 | 1 | 0.1uF |  |  |
| R4 | 1 | 1k |  | Install group on 4-pin headers |
| R9 | 1 | Open |  |  |
| C3 | 1 | Open |  | Optional low-pass filtering |
| C2 | 1 | Open |  | Optional low-pass filtering |
| R21, R22 | 2 | 1k |  |  |
| RT1, RT2 | 2 | 10k |  |  |
| RT3 | 1 | 1k |  |  |
| D4, D5 | 2 | Diode | 1N5711 |  |
| D1, D2 | 2 | LED Yel | LTL-4253 | Any color you prefer. |

INTEGRATED CIRCUITS

| U4, U5 | 2 | Opto-Isolated Solid-State Switch | TLP592 type or equivalent | Ron < 10 ohms |
| --- | --- | --- | --- | --- |

HARDWARE

| R1 - C1 R4 - C3 | 4 | 4-pin Socket Header for components | SSW-104-01-T-S | Install on both sides of component groups. |
| --- | --- | --- | --- | --- |
| U4, U5 | 4 | 3-pin female break-away socket strip |  | Install on both sides for opto switches. |
| J2 | 1 | 8-pin Socket Header | SSW-108-01-T-S |  |
| J1 | 1 | 2-pin Terminal Block screw type |  | Optional: Solder test leads (spring or alligator clips) directly to J1 pads. |
| TP1 | 1 | TestPoint | 5011 |  |

## DVM Advanced - 3 Ranges, Bipolar Input

PASSIVES

| C10, C11, C12 | 3 | 0.1uF |  | All caps Ceramic, >25V |
| --- | --- | --- | --- | --- |
| C13, C14, C15, C16 | 2 | 10uF |  |  |
| R1 | 1 | 825k | 1% for precision | Install group on 4-pin headers |
| R2 | 1 | 200k | 1% for precision |  |
| R3 | 1 | 200k |  |  |
| C1 | 1 | 0.1uF |  |  |
| R5 | 1 | 10k | 1% for precision | Install group on 4-pin headers |
| R6 | 1 | 30.1k | 1% for precision |  |
| R7 | 1 | 10k | 1% for precision |  |
| R8 | 1 | 10k | 1% for precision |  |
| R4 | 1 | Open |  | Install group on 4-pin headers |
| R9 | 1 | 1k |  |  |
| C3 | 1 | Open |  | Optional low-pass filtering |
| C2 | 1 | Open |  | Optional low-pass filtering |
| R11, R21, R22, R23 | 4 | 1k |  |  |
| RT1, RT2 | 2 | 10k |  |  |
| RT3 | 1 | 1k |  |  |
| D4, D5 | 3 | Diode | 1N5711 |  |
| D1, D2, D3 | 3 | LED Yel | LTL-4253 | Any color you prefer. |

INTEGRATED CIRCUITS

| U1 | 1 | Op Amp | LMC6082IN/NOP8 | Dual op amp, rail-to-rail output, +/-7.5V supply max, DIP |
| --- | --- | --- | --- | --- |
| U2 | 1 | Negative Rail Charge Pump | AD7660NP |  |
| U3 | 1 | Voltage Reference Shunt | LM4040xx |  |
| U4, U5, U6 | 3 | Opto-Isolated Solid-State Switch | TLP592 type or equivalent | Ron < 10 ohms |

HARDWARE

| R1 - C1 R5 - R8 R4 - C3 | 6 | 4-pin Socket Header for components | SSW-104-01-T-S | Install on both sides of component groups. |
| --- | --- | --- | --- | --- |
| U1, U2 | 2 | DIP-8 Socket | 4808-3000-CP |  |
| U4, U5, U6 | 6 | 3-pin female break-away socket strip |  | Install both sides for opto switches. |
| J2 | 1 | 8-pin Socket Header | SSW-108-01-T-S |  |
| J1 | 1 | 2-pin Terminal Block screw type |  | Optional: Solder test leads (spring or alligator clips) directly to J1 pads. |
| JP1 | 1 | 3-pin Header | TSW-103-08-T-S | 1 Shunts needed SNT-100-BK-T |
| TP1 | 1 | TestPoint | 5011 |  |

# Bonus Sections

## Revision & Notes

DVM Discovery User’s Guide: Rev 1a

The DVM Discovery provides real-world hands-on experiences to developing skills, understanding and intuition about measurement topologies.

Rick Faehnrich

https://www.linkedin.com/in/rick-faehnrich-2671a911/

## Learning Styles

Dive in any way that propels your learning with Self-Directed or Guided Learning (or a combination of both).

- Self-Directed Learning - Explore circuits entirely on your own.

- Guided Examples – Follow a walk-through of two DVM implementations:

  - 2-Range, Unipolar Input

  - 3 Range, Bipolar Input

- Mixed Mode - Get ideas from the Guided examples and follow-through on your own.

## Arduino Software

Here's the fast track steps of running a C program (sketch) on your Arduino Board.

- Download and install the Arduino IDE Software.
  https://www.arduino.cc/en/software

- Connect UNO Board to your computer
  https://docs.arduino.cc/hardware/uno-rev3/

- Open the Arduino IDE Software.

- Select Board Name
  Select UNO from drop down menu.

- Copy the program (sketch) to your Arduino folder and Open it. 
  DVM_discovery_basic_1.ino

- Click Compile Button
  Generates executable code, checks for errors.

- Click the Upload Button
  Uploads compiled code into UNO and begins running.

- Click the Serial Monitor
   Window displays output from PrintSerial statement.

## Excel DVM Design Calculator

A handy Design and Analysis Toolkit.
https://ecircuitcenter.com/DVM1/dvm-discovery.html

- Calculate gains, signal levels and ADC digital values.

- Modify, customize for your own projects.

## KiCAD Software

KiCad is a free and open-source software suite that provides a complete EDA toolset including schematic capture, PCB layout, 3D visualization, and circuit simulation. https://www.kicad.org/

## GitHub

The DVM Discovery Bd is an open-source project intended to make op amp learning as accessible and affordable as possible. You can download the documentation and KiCAD project files at https://github.com/rick-ecircuit.

## Feedback eCircuitCenter

Feedback on eCircuit Center

- How is the DVM Discovery Bd helpful for you?

- What needs clarity?

- Other comments, suggestions?

Send your thoughts on the feedback form at eCircuit Center.
https://ecircuitcenter.com/contact.htm

eCircuit Center strives to provide informal, hands-on and fun experiences that lead to a deeper, more intuitive understanding of circuit design and analysis.

## The Power of Play

- Key to analog success:  play, play, play with circuits!

- Analog circuits appear more inviting (and less intimidating) when playing and tinkering.

- Learning and passion grow when you

  - Ask “what if” questions

  - Play with component values - Predict the outcome!

  - Small discoveries and learning fuel motivation and the joy of learning.

## KiCAD Schematic